## Generate Aortic Valve Mesh in LS-PrePost

### Import CAD Surface Model

- Import File: File  Import  STEP File (aorticheartvalve3.stp)

  - Default Unit is mm! Use scale factor 0.1 for cm.

- Identify model parts in positive X-Y-Plane: View  Top, Unblank all other parts

- Save as Project File, “geometry.proj”

- Model Clean Up:

- Aorta: Remove unnecessary Vertexes on Surface Edges: Curve  Merge  Piecewise, Remove Raw Edge

- Stitch Surfaces of Leaflet and Aorta Surfaces: GeoTol  Stitch (Black Line shows stitched surfaces), Use non-manifold mode to stitch edges between 3 surfaces as well.

- Clean Up Commissure Surface: Add Missing Line on Commissure Part on Z=0 Plane by

  - Projecting Circle on Surface: Curve  Circle, Radius R=30,

  - GeoTol  Project  Conical Projection, Select Circle and Surface

  - Trim Commissure Surface on Z=0 Plane: GeoTol  Trim, Use Projected Circle as Trim Tool

- Save Project

### Create Leaflet Shell Mesh (Const Thickness)

- Unblank all but Leaflet Surfaces

- Divide Leaflet Surfaces:

  - Create Break Points on Surface Edges: Curve  Break  Parameter, N-Segment=2, Only Create Break Points, For Free Edge, Z-Plane Trajectory, Basal Attachment Edge and Belly Symmetry Curve.

  - Create Break Point on Belly Surface: Curve  Point, Click on Surface, Try to get centre of Surface. (0.7753508 cm, 0.5085122 cm, -0.3476466 cm)

  - Connect Break Points: Curve  Line  Point/Point

  - Project Break Lines onto Surfaces: GeoTol  Project  Normal Projection.

  - Subdivide Surfaces: GeoTol  Trim  MultiTrim

  - Stitch Surfaces and Remove unnecessary vertexes if needed!

  - Rename new Shell Surface Container

- Reflect Surfaces: GeoTol  Transf  Reflect, Plane  Norm Y, Copy

- Stitch Surfaces

- Mesh Leaflet: Mesh  NLineM  4LinesShell

  - Select Edges Counterclockwise starting on right edge (Element Direction will thus be upwards towards Free Edge)

  - Use Ratio 1.02 and 0.98 to create better quad ratios.

  - Contact Region Centre: 12x10

  - Contact Region Basal Attachment: 11x10

  - Belly Region adjacent to Contact Region: 11x10

  - Belly Region in Contact to Basal Attachment: 12x11

  - Belly Region Center: 12x10

  - Sum: 592*2*3=3552  Aspect Ratio < 1.48. With worst elements near basal attachment.

  - Check Element Direction: EleTol  EleEdit  Direction

  - Check Normal Direction: EleTol  Normal

  - Remove Duplicate Nodes: EleTol  DupNod, Show, Merge, Accept

  - Combine to one Part: EleTol  MovCop  Click All, PID=1

  - Optional: Smooth Surface: EleTol  Smooth, Click Smooth, Accept

  - Save as Project: geometryLeafletShell

- Create Shell Section: Model  Keywrd  Section, Shell, T1=0.5 Enter

- Assign Section to Part and Delete old Parts: Model  Keywrd  Part, Section=1

- Show Thickness: Model  Appear  Thick, AllVis, Done

### Create non-uniform thickness Leaflet

- Create Areas of different thickness: EleTol  MovCop

  - For Belly, For Basal Attachment, For Free Edge

  - Intermediate Layers in between areas above for gradual change of thickness.

- Smooth Leaflet Surface (optional): EleTol  Smooth

  - Lock Nodes  Click Nodes on corner between free edge and Commissure and at bottom of symmetry line  3 points!

  - Select Elements  Click all elements

  - Smooth Option:  Untick Feature Angle, Lock Boundary and Project Back

  - Iterations = 20 (Maybe less)

  - Smooth, Accept

  - Smooth Surface Deviations with respect to original geometry

    - At Symmetry Line: Between 0 and 0.074 cm. Increasing towards free edge.  Max. 3,1% with respect to Heart Valve Diameter. Average: 0.023 cm  0.96%

    - At Directrix Curve: Between 0 and 0.07 cm. Decreasing towards Symmetry Curve  Max 2,9% with respect to Heart Valve Diameter. Average: 0.036 cm  1,5%

    - At Free Edge: Between 0 and 0.066 cm. Increasing towards Symmetry Line  Max 2,75% with respect to Heart Valve Diameter. Average: 0.0067 cm  0,28%

    - At Basal Attachment: Between 0 and 0.07 cm. Decreasing from Intersection Point of Commissure and Directrix Curve towards Free edge and symmetry Line.  Max. 2.75%. Average:

- Assign different thickness: EleTol  EleEdit  Modify  Uniform Thickness to Shell, By Part. Use 0.05 mm difference in thickness for adjacent layers.

- Create Solid from Shell: Mesh  EleGen  Solid  Shell_Thickness, Segment = 3. Unblank Delete Shell.

- Smooth Free Edge (Optional)  EleTol  Smooth

  - Select Elements  ByPath  Choose Elements on Free Edge on Fibrosa and Ventricularis Side. Only one Layer!

  - Lock Nodes  By Part  Choose all. Click on Rm and choose nodes on free edge of Fibrosa and Ventricularis. Don’t Remove the points on the Commissure of the free edge! Iterations =3.

  - Adjust nodes on Commissure: EleTol  Transf  Translate  N1-N2, Distance = 0.016 (First Picture below)

  - Adjust nodes on Symmetry Line at Basal Attachment: Distance 0.004 in upward direction (N1 to N2) See Picture below. 0.002 in direction towards inner Belly.

### Create Solid Valve Surface

- Create Shell Mesh for the upper and lower faces of the volume mesh:

  - Mesh  EleGen  Shell by Solid Face

  - Create 4 Surfaces for Fibrosa (Use original shell to see where the boundary of each part must be)

  - Create 4 Surfaces for Ventricularis

  - Create 4 Surfaces for Basal Attachment

  - Create 6 Surfaces for Free Edge

- Create Surfaces from Shell mesh

  - Surf  FitSuf  Select by Part

  - MaxTol = 0.0001, Smoothness decreased to min (left position of slider)

  - Preview  Fit  Apply

  - Do this for all 18 Parts

  - Check Each Surface and make sure that the boundaries are represented exactly!

  - For the Free edge a different approach must be used. Create Wires like in the picture below using Curve  Line  Point to Point

Surf  NSide  Select by Area, Use Simple Fit

- Add Vertices on the corners of every Surface: Curve  Break  Click Edge, Mayb you must decrease the Max angle.

- Merge edges to remove vertices on the edges which are no corner points: Curve  Merge  Piecewise, Remove Raw Edge.

- Stitch the surfaces one by one: GeoTol  Stitch

### Create Aortic Root Surfaces

- Create new Sinus

  - Create three circular arcs (red, blue, green) using the original Sinus geometry (Brown Surface) and the newly generated Valve surfaces.

Curve  CircArc  3 Points

Make sure to use the highest valve point for the blue arc.

  - Trim The three arcs: GeoTolTrim  Standard

  - Create Sinus Surfaces: Surf  N-Side  Continuity0  AutoClose. Make sure to include the small edge on the free edge

  - Stitch surfaces: GeoTol  Stitch  Non-manifold mode. Pink lines show edges stitched to more than one other surface. Here three surfaces are stitched together at one edge.

  - Create 4 Circles using all the original shell elements

  - Mirror the Created Shell: GeoTol  Transform  Rotate  Use whole shell as entity  Direction=z  Angle 120°  Copy Instances = 2

  - Create two more circles for the bottom of the leaflet (brown) and the commissure of the leaflet (Yellow): Curve  Circle  3 points.

  - Trim the circles for the interleaflet triangle. Use Multi Trim Option

  - Create the interleaflet surfaces: Surf  Nside  Continuity 0, untick all checkboxes. You might have to change the order of the members in the edge list to get the correct result.

Create the part between the leaflets the same way but this time use auto close.

  - Rotate/Mirror the interleaflet triangles and Stitch the surfaces.

  - Use original Shells to generate the red and the blue straight lines. Use the lowest point on the leaflet as first point of line and lowest point of original shell as second point. Trim the brown and green circle using the red and blue line. And create the surface using the N-Side feature as before.

  - Repeat step k for the Aorta Part

  - Use the circles at the top and bottom of the geometry and create inlet and outlet using N-Side Surface

  - Delete unnecessary parts. Only keep one Sinus, Leaflet, Interleaflet triangle, as well as 1/3 of top and bottom part.

### Create Smoothed Aortic Root Surfaces

- Stitch interleaflet triangle and inlet region (grey region)

- Stitch aortic region (grey)

- Mesh the surfaces from 1. And 2. Using Mesh  AutoM  Mixed  Compute  Mesh. Create one Mesh for grey parts of step 1 and one mesh for grey parts of step 2

- Smooth created meshes.

- Use Surf  Fit Surf  MaxTol = 0.0001, Smoothness = min, Preview  Fit and Apply on the two generated meshes from step 4 separately.

- Create a ref Plane and trim the Interleaflet triangle

Smooth Surface Deviations with respect to original geometry:

- At Commissure Symmetry Line

Max: 0.036 cm  1.4%*, Average: 0.0098 cm  0.41%*, biggest difference above sinus

- At Sinus Symmetry Line

Max: 0.047 cm  1,96%*, Average: 0.00651 cm  0.27%*, biggest difference above sinus

- Use Merge Curve and Break Curve to eliminate vertices on edges and create vertices on corners.

Create The Aortic Root Mesh for FEM

Rotate/Mirror the Shell Meshes representing the basal attachment area. Blank all other shell and volume meshes, since they will cause problems connecting boundary nodes automatically when meshing.

Start Meshing the aortic root parts. Create one part for the annulus (green Part) and one for the Sinus and ascending aorta. Mesh  AutoM  Size  Mixed  Compute  Connect Boundary Nodes  Mesh and Accept. You might have to increase the Merge Tolerance

Oberflächen für gmsh anpassen.

- Zwei halbe Segel wie in dem Bild durch Reflect und Rotate Mirror erzeugen

- Inlet und Outlet dritteln. Dazu z-Achse erstellen, Punkt auf Achse zeichnen und auf Oberfläche projizieren. Danach zwei Linien vom Kreismittelpunkt zur Aorta ziehen, Mergen und die Oberfläche trimmen. Unnötige 2/3 Flächen löschen.

- Durch Break Curve Vertex zwischen den beiden geraden Kanten erstellen.

- File  SaveAs  SaveGeomAs  STEP

### VI Fluid Mesh Generation in Gmsh

- Open Gmsh.exe

- Import Step File: File  Open  Choose File

- Adjust graphical representation: Tools  Options

  - Geometry  Visibility  Surfaces

  - Mesh  General  Max element size = 0.1

  - Mesh  General  Visibility  2D Faces

- Create Basic Mesh: Mesh  1D  2D

- Adjust Mesh Density: Mesh  Define  Size at Points  Click Points and enter element size

  - When you are asked to create new Geo file click Create new .geo file.

  - Open the geo file in notepad++ and update.

  - Adjust the comments in the geo file such that you know which region is related to which mesh density

- Export Mesh: File  Export .key