# Generate Heart Valve Geometry

## Import Model:

- File  Import  Step: D:\Simulation\1_CAD Modell CREO\HeartValve_20250613\AorticValveCAD.stp

- Scale Factor = 0.1  Conversion mm in cm

- Identify all parts in positive x- and y -Direction  1/6 of complete model. Include Commissure Region in positive x-y- direction (violet).

- Delete all other parts

- Remove unnecessary vertices: Curve  Merge  Piecewise, Remove Raw Edge  Select Sinus upper edge which is in contact with the free edge

- Stitch Surfaces:  GeoTol  Stitch  Choose all aortic root surfaces, Repeat with 2 leaflet surfaces.  Click non-manifold mode for stitching surfaces at edges where 3 or more Surfaces come in contact.  All Lines between surfaces should be black (two surfaces) or violet (three surfaces). Outer Edges are light blue.

- Save as project: D:\Simulation\2_Geometrie LS-DYNA\geometry_20250613\AorticValveGeometry_1.proj

## Create Smooth Leaflet Surfaces:

- Hide aortic root surfaces

- Reflect the two leaflet surfaces: GeoTol  Transf  Reflect  Plane: NormY  Copy

- Curve  Break  Method: Parameter  Only Create Break Points.

  - Use N Segment = 10 for each of the Free Edge Lines

  - Use N Segment = 10 for each of the Belly-Coaptation Zone Edges

  - Use N Segment = 5 for the two Commissure Lines & the Symmetry Line in the y-Plane. For z > 0

  - Use N Segment = 10 for the Symmetry Line in the y-Plane for z<0

  - Use N Segment = 15 for the two Basal Attachment Edges.

Higher values reproduce the original surface better but are more angular. Lower values ensure a rounder geometry with fewer kinks.

- Curve  Spline  Untick Piecewise, Tick Sort Points

  - Use all Points on the free edge, expect the one on the symmetry line, for the smoothed Free Edge. (red line)

  - For the Commissure & Basal Attachment Lines (green lines) use all points except the 2 points at z=0 to create a smooth transition between Commissure and Basal Attachment. Don’t forget the point on the symmetry line!

RefGeo  Plane  Create two planes. One with Normal 0,0,1 and one with 0,1,0

Trim the red and green line at the two planes to create the missing vertices

- Use all point on the z=0 surface except the one at the symmetry line (y=0) to create a smooth Directrix Curve of the leaflet between belly and coaptation area.

- Trim the line to create the missing vertex on y=0 plane.

- GeoTol  Trim  Use Curves as Trim Tool and trimmed entity to trim curves at intersections with other curves

- Create a Spline. Use all points on the symmetry line except the one directly below the free edge.

- Surf  N-Side  Click all 4 edges of the coaptation region in the positive x-y Plane. Repeat this with the 3 edges for the Belly in the positive x-y- Plane.

- Save as project: D:\Simulation\2_Geometrie LS-DYNA\geometry_20250613\AorticValveGeometry_2.proj

## Create structured Shell Mesh

- Curve  break  Parameter, N Segment = 2, Only Create Break Points for Free Edge, Coaptation-Line, Symmetry Line and Basal Attachment Line.

- Curve  Point  X=0.7753508 cm, Y =0.5085122 cm, Z=-0.3476466 cm

- GeoTol  Project  Destination Belly Surface, Source is the created Point

- Curve  Line  Point/Point. Create lines from Break Point on edges to Point on Belly Surface. Create Line from free edge middle to Coaptation Point Middle.

- GeoTol  Project. Project all 4 lines onto the Leaflet Surfaces.

- GeoTol   Trim  Trim Belly Surface and Coaptation Surface by projected Lines as Trim Tools.

- GeoTol  Transform  Reflect, Choose all 5 Trimmed Surfaces, Tick Copy. Choose NormY as plane.

- Mesh  N-Line Mesh  Type: 4 Lines Shell  Click edges in counter-Clockwise direction starting with the right edge. Click Mesh It and Accept. Repeat this for all 10 surfaces. Use N1 =N2 = N3 = N4 = 5 for coarse mesh with 250 elements per leaflet  750 elements for complete valve. Use N1-N4 = 10 for medium mesh with 1000 elements per leaflet  3000 elements for complete valve. Use N1-N4 = 20 for fine mesh with 4000 elements per leaflet  12000 for complete valve.

- EleTol  Element Editing  Direction  Check if all arrows point towards the free edge  This is the local positive x-Direction of the elements.

- EleTol  MovCop  Select all 10 mesh parts and Move with PID = 1. Apply.

- EleTol  Duplicate Nodes  Show Dup Nodes  Merge Dup Nodes. Make Sure that all nodes are included. Otherwise increase the tolerance and check again.

- EleTol  Smooth  Use default values

- Application  Model Checking  General Checking. Perform Quality Check for Shell Mesh

  - Aspect Ratio = 1.03 - 1.48; 1.03-1.53; 1.01-1.57

  - Warpage = 0.0593 – 6.23; 0-3.27; 0-1.65

  - Min Quad Angle = 70.9-89.5; 68.2-89.8; 63.7-89.9

  - Max Quad Angle = 90.5-124; 90.3-125; 90.1-125

  - Skewness = 0.0524-16.1; 0.0135-21.2; 0.0035-26.1

  - Jacobian = 0.778-0.992; 0.786-0.996; 0.79-0.998

  - Feature Angle = 1.67-20.5; 0.745-13; 0.245-9.44

- For the medium and fine mesh: EleTol  Smooth  Select all Elements, Lock the nodes in the picture. Use Options like in the picture to smooth the belly and coaptation areas, while keeping the symmetry line and the coaptation belly Line.

- Save as project: D:\Simulation\2_Geometrie LS-DYNA\geometry_20250613\AorticValveGeometry_3.proj

## Create non-uniform Thickness Distribution.

- EleTol  MovCop. Choose elements that have the same thickness and give them a separate part ID. Check Picture 6 for the chosen Thickness Levels. The free Edge is thinner than the coaptation Zone which lies approx. 1-2 mm below the Free Edge. The Belly is the thinnest part. The Basal Attachment is comparable to the coaptation area.

For the middle Mesh first create the same spatial distribution, then create Transition Zones between each of the three layers.

- Model  Keyword  Section  Shell  T1 = Insert Thickness of layer & press enter. Title should include the thickness. Repeat for every thickness level that should be created. Belly (green, Part 3) = 0.42 mm = 0.042 mm, Coaptation & Basal Attachment (red, Part 1) = 0.53 mm = 0.053 cm, Nodulus = 1.05 mm = 0.105 cm. Transition Zone (blue, Part 2) = 0.475 mm = 0.0475 cm

For the middle mesh: Use Additional: Transition Zone (yellow, Part 4) = 0.04475 cm, Transition Zone (Brown, Part 5) = 0.05025 cm

For the fine mesh: Use Additional: Transition Zone  (Red, Part 6) = 0.043375, Transition Zone  (Dark Blue, Part 7) = 0.046125, Transition Zone  (Green, Part 8) = 0.048875, Transition Zone  (Yellow, Part 9) = 0.051625

| Part 1 | Red | Coarse | 0.053 cm |
| --- | --- | --- | --- |
| Part 2 | Blue | Coarse | 0.0475 cm |
| Part 3 | Green | Coarse | 0.042 cm |
| Part 4 | Yellow | Medium | 0.04475 cm |
| Part 5 | Brown | Medium | 0.05025 cm |
| Part 6 | Red | Fine | 0.043375 cm |
| Part 7 | Dark Blue | Fine | 0.046125 cm |
| Part 8 | Green | Fine | 0.048875 cm |
| Part 9 | Yellow | Fine | 0.051625 cm |

- Model  Keyword  Part  Apply correct SECID for every part.

- Model  Appear  Tick option Thick and click AllVis. Check if the Thickness Distribution is correct.

- Mesh  EleGen  Solid  Solid by: Shell Thickness, Untick Delete Shell, Enter number of segments through thickness = 1. Create.

- Save as project: D:\Simulation\2_Geometrie LS-DYNA\geometry_20250613\AorticValveGeometry_4.proj

## Create Nodulus

- EleTol  Measure  DistN2N  Select the node, where the centre of the nodulus should be located. Click the node on the other side of the valve to see the actual thickness. Here: second node beneath the free edge at the symmetry line: coarse: 0.050631 cm, For middle: 0.0519651 cm, For Fine: 0.0523125 cm

- Calculate difference distance between nodulus thickness and actual thickness: Coarse: 0.105 cm – 0.050631 = 0.054369 cm

Middle: 0.105 cm – 0.0519651 = 0.0530349 cm

Fine: 0.105 cm – 0.0523125 = 0.0526875 cm

- EleTol  Transform  Translate  Direction: N1-N2  Click Nodes 1 and 2 for the direction. Click the point you want to translate on the fibrosa. Here: Second vertex below free edge at symmetry line. Tick Copy Node Option, No. of copies: 1  Tran+

- Curve  Spline  Interpolate Points, untick Piecewise. Select the points to create the nodulus wire. See the picture.

- Use GeoTol  Trim  Multi Trim and trim all splines at each other

- Surf  N-Side  Create 4 Nodulus surfaces by clicking the edges.

- Use Mesh  AutoMesh and mesh all 4 surfaces. Click compute and reduce the size of the mesh by factor 10 to have a very fine mesh

- EleTol  Transform  Project  Plane: To Mesh, Vector Projection with -1,0,0, Tick Positive Direction Only, Select Nodes and Choose all elements of the nodulus mesh

- Save as project: D:\Simulation\2_Geometrie LS-DYNA\geometry_20250613\AorticValveGeometry_5.proj

## Create finished heart valve mesh

- Create elements through thickness:

  - For coarse and fine mesh: Mesh  EleEdit  Split/Merge  Solid  Last Option  Click Heart Valve Mesh. Note: This way you can only create 1,2,4, 8 etc. elements through the thickness! If you want other values use the following approach!

  - For Middle: Create two shell Sets, one for the fibrosa surface and one for the ventricular surface: Mesh  Element Generation  Shell  Shell By: Solid Face. Select all Surface elements of the respective surface: See Picture on the left.

Then create a solid between the two shell surfaces with the desired number of elements through the thickness:  Mesh  Element Generation  Solid  Solid By: Two Shell Sets, Tick new node, Click Set 1 and choose Fibrosa surface, Use the red point on Fibrosa as N1 and the green point as N2, Use Ventricular Surface as Set2, Red Node on Ventricularis as N1 and Green one as N2. Choose number of segments. See picture on the right!

- Check Mesh Quality: Medium Mesh, Fine Mesh

  - Minimum Angle: 51.94-89.37; 53.56-89.65

  - Maximum Angle: 90.631-124.7; 90.38-124.93

  - Distortion Index: 0.5857-0.991537; 0.6025-0.9937

  - Aspect Ratio: 1.93-5.8; 1.40-3.66

- Create other two leaflets: EleTol  Transform  Rotate, 120 degress, Copy Elements, No. of copies = 2

- Don’t forget to delete the node used for Nodulus Creation!

- Save as project: D:\Simulation\2_Geometrie LS-DYNA\geometry_20250613\AorticValveGeometry_6.proj

- Save as keyword: D:\Simulation\2_Geometrie LS-DYNA\geometry_20250613\AorticValveMesh_Coarse.k